1. Main Equipment Details

- Tag: PP01-362-V01

- Name: Fuel Gas Knockout Drum

- Dimensions:

  - ID: 2200 mm

  - Length (T/T): 5500 mm

  - OD: 5500 mm (Transcript mentions OD 5500, though likely refers to Length, ID is confirmed as 2200).

- Design Parameters:

  - Design Pressure: 14 bar (Note: Transcript says "15 14 bar", context of PSV suggests 14).

  - Operating Pressure: 9 to 10.4 barge.

  - Operating Temperature: 23 to 72°C.

- Material: Carbon Steel + 3mm Corrosion Allowance.

- Internal: Epoxy Ceramic Coating (No insulation).

- Trim Number: PP01 362 V1 TX1 V 03 F9.

2. Process Lines & Correlations

A. Inlet Lines (Feed)

Two lines join the vessel at Nozzles N4 and N5.

- Source 1: Fuel Gas from DS1 Scraper Receiver.

  - Line Number: 16” PP01-361-GF0014-P03-F9-PP

  - Nozzle Connection: N5 (12” nozzle).

- Source 2: Fuel Gas from DS3 Scraper Receiver.

  - Line Number: 16” PP01-361-GF0031-P03-F9-PP

  - Nozzle Connection: N4 (12” nozzle).

- Inlet Configuration: Both lines have a Check Valve -> 16” Ball Valve -> Reducer (16” to 12”) -> Connection to Vessel.

B. Gas Outlet (Top)

- Nozzle: N2 (16”).

- Primary Line: 16” PP01-362-GF02-B01-E9-PP.

  - Spec Change: Starts B01-E9.

  - Components: 16” Blind Valve -> Junction of PG (Gauge) -> HV15 (Valve).

- Destinations (Splits into 3 paths):

  - To Fuel Gas Treatment Package:

    - Line continues as 16” to NPG-PP01-PRP-ID-09-01.

  - To HP Flare Header (Blowdown Line):

    - Line: 6” PP01-362-GF09-B01-E9-PP.

    - Valves: HV061 (6” Ball) -> BDZV-01 (Control Valve).

    - Transition: Reducer 6” to 8” -> HV029 (8” Ball).

    - Spec Change: B01-E9 to B01-B8.

    - Final Line: 8” PP01-500-VK-026-B01-B8 -> HP Flare Header (NPG-PPP01-PRP-E-021).

  - To HP Flare Header (Pressure Control):

    - Line: 8” PP01-362-GF03-B01-E9-PP.

    - Main Path: HV16 (8”) -> PV02 (Globe/Control Valve) -> Reducer (8” to 10”) -> HV21 (10”).

    - Drains: HV20 (1”) and HV17 (1”) located around the control valve/reducers.

    - Bypass Line: Parallel to PV02, contains 4” HV18 and 4” HV0019.

    - Spec Change: B01-E9 to B01-B8 after HV21.

    - Final Line: 10” PP01-500-VK-00-B01-B8 -> HP Flare Header (NPG-PPP01-PRP-021-01).

C. Liquid Outlet / Drain (Bottom)

- Nozzle: N3.

- Line: 4” PP01-512-PK-06-B01-M8-PP (Originates as B01-E9).

- Destination: Condensate Storage Vessel (NPG-PP01-PRP-ID-2401).

- Main Control Path:

  - HV27 (4” Gate) -> EZV01 (Control Valve) -> HV28 (4” Gate).

- Bypass/Low Flow Path:

  - Junctions before EZV01 and after HV28.

  - Line: 2” -> Reducer (2” to 1.5”) -> HV004 (1.5” Ball) -> HV29 (1.5”) -> LV01 (Control Valve) -> HV30 (1.5”) -> Reducer (1.5” to 4”).

  - Correction from transcript: There is a mention of HV32 (2”) leading into this section.

- Drains: HV065 (2” Gate, NC) and HV30 (0.5”).

- Instrumentation: CIT-01 (Conductivity) on a 2” blind junction.

D. Manual Drain / UC Line

- Nozzle: N9 (2”).

- Valves: HV02 (2” Gate) -> HV01 (2” Check) -> Reducer (2” to 0.5”).

- Drain: HV51 (1” Drain) located between nozzle and check valve.

- Destination: UC (Bucket type structure).

- Line: 2” PP01-567-GF02-B03-F9.

3. Safety Relief System (PSV)

- Nozzle: N1 (14”).

- Common Line: 14” PP01-362-GF01-B01-E9-PP.

- Junction Line: 14” PP01-362-GF04-P01-E9-PP.

- PSV-001 Path:

  - Valves: HV04 (14” Ball) -> Reducer (14”x8”) -> PSV-001 -> Reducer (10”x16”) -> HV39 (Ball).

  - Drains: HV5 (1”) upstream, HV0038 (1”) downstream.

  - Line: 16” PP01-500-VK-66-B01-B8.

- PSV-002 Path:

  - Valves: HV06 (14” Ball) -> Reducer (14”x8”) -> PSV-002 -> Reducer (10”x16”) -> HV041.

  - Drains: HV07 (1”) upstream, HV041 (1” - duplicate tag in transcript, likely drain) downstream.

  - Line: 16” PP01-500-BK-08-B01-B8.

- Destination: HP Flare Header (NPG-PPP-01-PRP-021-001).

- PSV Specs:

  - Set Pressure: 14 bar.

  - Size: 8T10.

  - Case: DS3 PCV Failure.

4. Instrumentation & Level Bridles

The vessel has multiple level bridles connected via nozzle pairs (Upper and Lower).

Group 1: Bottom Level Instrumentation

- Common Junction Line: 1” PP01-512-PK-54-B01-M8-PP.

- Nozzles N18A & N18B:

  - Solenoids: HV024 (2”), HV025 (2”).

  - Instrument: LG-001 (Level Gauge).

  - Valves: HV26 (1”), HV27 (1”).

  - Line: 1” PP01-512-PK-076-B03-F9-PP.

- Nozzles N7A & N7B:

  - Solenoids: HV22 (2”), HV0023 (2”).

  - Instrument: LIT-001 (Level Transmitter).

  - Valves: HV28 (1” Drain), HV49 (1”), HV0029 (1” Gate).

  - Line: 1” PP01-512-PK-027-B03-F9-PP.

- Nozzles N10A & N10B:

  - Solenoids: HV30 (2”), HV31 (2”).

  - Instrument: LIT-03.

  - Valves: HV003 (1” Drain), HV33 (1” Gate), HV51 (1” Gate).

  - Line: 1” PP01-512-PK-078-B03-F9-PP.

Group 2: Top/Interface Level Instrumentation

- Common Header Line: 1” PP01-512-PK-079-B03-F9-PP.

- Nozzles N12A & N12B:

  - Solenoids: HV38 (2”), HV39 (2”).

  - Instrument: LZT-002A.

  - Valves: HV40 (1” Drain), HV41 (1” Gate).

- Nozzles N6A & N6B:

  - Solenoids: HV34 (2”), HV35 (2”).

  - Instrument: LZT-002C.

  - Valves: HV44 (1” Gate), HV45 (1” Gate).

  - Line: 1” PP01-512-PK-0081-B03-F9-PP.

- Nozzles N1A & N1B (Refers to L1A/L1B):

  - Solenoids: HV36 (2”), HV37 (2”).

  - Instrument: LZT-002B.

  - Valves: HV42 (1” Drain), HV043 (1”).

  - Line: 1” PP01-512-PK-080-B03-F9-PP.

- Common Drains/Isolations for Group 2:

  - HV52 (1” Gate).

  - HV56 (1” Drain).

Group 3: Pressure Instrumentation (Top Nozzles)

- N13: HV011 (2” Solenoid) -> PZT-01A, PI-1A.

- N14: HV12 (2” Solenoid) -> PZT-01B, PI-01B.

- N15: HV13 (2” Solenoid) -> PZT-01C, PI-01C.

- N16: HV14 (2” Solenoid) -> PIT-02, PIC-02 (Controls PV02).

5. Maintenance / Miscellaneous Nozzles

- N17: 3” Ball Valve HV03 (Manway/Vent).

- N8: 3” Gate Valve HV0064.

- N14 (Secondary reference): Transcript mentions "N14 is a 3-in ball valve HV3" at the end, contradicting the pressure transmitter section. Based on standard P&ID layouts, N17 is likely the maintenance/drain valve, and N14 is the instrument nozzle.

Nozzle N1: Safety Relief System (PSV Header)

This nozzle serves as the connection point for the safety relief valves, discharging gas to the flare header in overpressure scenarios.

- Nozzle Size: 14 inch

- Main Header Line: 14” PP01-362-GF01-B01-E9-PP

- Correlations & Split: The line splits from the nozzle into two parallel relief paths (PSV-001 and PSV-002).

Path A (PSV-001):

- Valves:

  - HV04: 14” Ball Valve (Inlet isolation).

  - HV5: 1” Drain Valve (Located between HV04 and PSV).

  - PSV-001: Safety Relief Valve.

    - Set Pressure: 14 bar.

    - Size: 8T10.

    - Case: DS3 PCV Failure.

  - HV39: 16” Ball Valve (Outlet isolation).

  - HV0038: 1” Drain Valve.

- Piping Config: Reducer (14” to 8”) at inlet → Reducer (10” to 16”) at outlet.

- Exit Line: 16” PP01-500-VK-66-B01-B8.

Path B (PSV-002):

- Valves:

  - HV06: 14” Ball Valve (Inlet isolation).

  - HV07: 1” Drain Valve.

  - PSV-002: Safety Relief Valve (Same specs as PSV-001).

  - HV041: 16” Ball Valve (Outlet isolation).

  - HV41 (Drain): 1” Drain Valve (Transcript repeats tag HV41 for drain).

- Piping Config: Reducer (14” to 8”) at inlet → Reducer (10” to 16”) at outlet.

- Exit Line: 16” PP01-500-BK-08-B01-B8.

Final Destination: Both paths discharge to HP Flare Header (NPG-PPP01-PRP-021-001).

Nozzle N2: Main Gas Outlet

This is the primary gas outlet at the top of the vessel. The line splits into three distinct functions: Process Gas, Blowdown, and Pressure Control.

- Nozzle Size: 16 inch

- Primary Line: 16” PP01-362-GF02-B01-E9-PP

- Immediate Connection: 16” Blind Flange and a PG (Pressure Gauge) connection via HV15.

Branch 1: To Fuel Gas Treatment

- Line: Continues as 16” pipe.

- Destination: Fuel Gas Treatment Package (NPG-PP01-PRP-ID-09-01).

Branch 2: Blowdown / Depressurization

- Line: 6” PP01-362-GF09-B01-E9-PP.

- Valves:

  - HV061: 6” Ball Valve.

  - BDZV-01: Control Valve.

  - HV029: 8” Ball Valve (Located after reducer).

- Configuration: Reducer increases line from 6” to 8”.

- Exit Line: 8” PP01-500-VK-026-B01-B8.

- Destination: HP Flare Header (NPG-PPP01-PRP-E-021).

Branch 3: Pressure Control to Flare

- Line: 8” PP01-362-GF03-B01-E9-PP.

- Main Control Path:

  - HV16: 8” Ball Valve.

  - PV02: Globe/Control Valve.

  - HV21: 10” Ball Valve.

  - Drains: HV17 (1”) and HV20 (1”).

- Bypass Path: Parallel line containing 4” HV18 and 4” HV0019 (Globe).

- Configuration: Reducer increases line from 8” to 10” before HV21.

- Exit Line: 10” PP01-500-VK-00-B01-B8.

- Destination: HP Flare Header.

Nozzle N3: Liquid Outlet / Condensate Drain

Located at the bottom of the vessel, this nozzle drains condensate to the storage vessel.

- Nozzle Origin: N3 (Transcript notes connection starts as B01-E9 spec).

- Main Line: 4” PP01-512-PK-06-B01-M8-PP.

- Destination: Condensate Storage Vessel (NPG-PP01-PRP-ID-2401).

Main Flow Path:

- HV27: 4” Gate Valve.

- EZV01: Control Valve.

- HV28: 4” Gate Valve.

- HV065: 2” Gate Valve (Normally Closed Drain) located before EZV01.

Low Flow / Level Control Bypass:

- This path exists parallel to the main control valve EZV01.

- Junction: Starts at HV32 (2”) junction.

- Configuration:

  - Reducer (2” to 1.5”).

  - HV004: 1.5” Ball Valve.

  - HV29: 1.5” Valve.

  - LV01: Control Valve (1.5”).

  - HV30: 1.5” Valve (with a 0.5” drain HV30 nearby).

  - Reducer (1.5” back to 4”).

  - Rejoins main line before HV28.

Instrumentation:

- CIT-01: Conductivity Indication Transmitter connected to a 2” Blind Flange junction on the outlet line.

Nozzle N4: Fuel Gas Inlet (Source 2)

One of the two main feed lines entering the vessel.

- Nozzle Size: 12 inch

- Source: Fuel Gas from DS3 Scraper Receiver.

- Line Number: 16” PP01-361-GF0031-P03-F9-PP.

- Connection Detail:

  - The line is 16” coming from the source.

  - It contains a Check Valve followed by a 16” Ball Valve.

  - It passes through a Reducer (16” to 12”).

  - It enters the vessel at Nozzle N4 (which is 12”).

Nozzle N5: Fuel Gas Inlet (Source 1)

The second main feed line entering the vessel.

- Nozzle Size: 12 inch

- Source: Fuel Gas from DS1 Scraper Receiver.

- Line Number: 16” PP01-361-GF0014-P03-F9-PP.

- Connection Detail:

  - The line is 16” coming from the source.

  - It contains a Check Valve followed by a 16” Ball Valve.

  - It passes through a Reducer (16” to 12”).

  - It enters the vessel at Nozzle N5 (which is 12”).

Nozzle N6 (N6A & N6B): Interface/Level Bridle

This nozzle pair forms a bridle connection for level instrumentation, specifically connecting to a transmitter denoted as LZT-002C.

- Configuration: Two nozzles (A and B) create a loop for the instrument.

- Valves:

  - N6A Connection: Connects to a 2” Solenoid Valve HV34.

  - N6B Connection: Connects to a 2” Solenoid Valve HV35.

- Piping & Isolation:

  - Line Number: 1” PP01-512-PK-0081-P03-F9-PP.

  - HV44: 1” Gate Valve located before the LZT-002C instrument connection.

  - Blind: A 2” blind flange is located before HV44.

  - HV45: 1” Gate Valve located after the N6B/HV35 connection.

- Correlations:

  - This bridle line connects to a common header line (1” PP01-512-PK-079-B03-F9-PP).

  - It joins the header before the common drain valve HV0056.

Nozzle N7 (N7A & N7B): Level Transmitter Bridle

This nozzle pair is dedicated to level monitoring, connecting to LIT-001 (Level Indication Transmitter).

- Configuration: Two nozzles forming a bridle.

- Valves:

  - N7A Connection: Connects to a 2” Solenoid Valve HV22.

  - N7B Connection: Connects to a 2” Solenoid Valve HV0023.

- Piping & Isolation:

  - Line Number: 1” PP01-512-PK-027-B03-F9-PP.

  - HV28: 1” Drain Valve located before HV22.

  - HV0029: 1” Gate Valve located after the N7A connection junction.

  - HV50: Another 1” valve found further down this line.

- Correlations:

  - This assembly connects to a common instrument header that eventually links with the N18 (LG-001) assembly.

  - The combined line transitions to line number 1” PP01-512-PK-54-B01-M8-PP.

Nozzle N8: Maintenance/Drain Connection

A standalone nozzle, likely used for maintenance or direct vessel drainage/access.

- Size: 3 inch.

- Valve: HV0064 (3” Gate Valve).

- Connection: The valve is directly connected to the nozzle; no further complex instrumentation bridle is described for this specific nozzle in the text.

Nozzle N9: Manual Drain to UC

This is a bottom/manual drain connection leading to a containment structure (UC).

- Nozzle Size: 2 inch.

- Line Number: 2” PP01-567-GF02-B03-F9.

- Valve Arrangement (in order from nozzle):

  - HV02: 2” Gate Valve (Isolation).

  - HV51: 1” Drain Valve (Located between HV02 and the Check Valve).

  - HV01: 2” Check Valve (Prevents backflow).

  - Reducer: Reduces line size from 2” to 0.5”.

- Destination: Terminates at UC (described as a "bit type structure," likely a bucket or pit drain).

Nozzle N10 (N10A & N10B): Level Transmitter Bridle

This nozzle pair connects to LIT-03 (Level Indication Transmitter).

- Configuration: Two nozzles forming a bridle.

- Valves:

  - N10A Connection: Connects to a 2” Solenoid Valve HV30.

  - N10B Connection: Connects to a 2” Solenoid Valve HV31.

- Piping & Isolation:

  - Line Number: 1” PP01-512-PK-078-B03-F9.

  - HV003: 1” Drain Valve connected before HV30.

  - Blind: A 2” blind flange is located before the LIT-03 instrument.

  - HV33: 1” Gate Valve located after the connection of N10B.

  - HV51: Another 1” Gate Valve further down the line.

- Correlations:

  - The piping spec changes at a junction from B03-F10 to B01-M8.

  - This line eventually joins the common header 1” PP01-512-PK-54-B01-M8-P, which is the same header shared by the N7 and N18 assemblies.

Nozzle N11 (N11A & N11B): Level Transmitter Bridle

This nozzle pair connects to the Level Transmitter LZT-002B.

- Configuration: Two nozzles (A and B) forming a bridle loop.

- Valves:

  - L11A Connection: Connects to 2” Solenoid Valve HV36.

  - L11B Connection: Connects to 2” Solenoid Valve HV37.

- Instrumentation: LZT-002B (Level Transmitter).

- Piping & Isolation:

  - Line Number: 1” PP01-512-PK-080-B03-F9-PP.

  - HV42: 1” Drain Valve located between the LZT instrument and the L11A junction.

  - Blind: A 2” blind flange is located between the line and the LZT instrument.

  - HV043: 1” Gate Valve located after the connection of L11B.

- Correlations:

  - This line connects to the common instrument header line 1” PP01-512-PK-079-B03-F9-PP.

  - The connection point is before the common drain valve HV0056.

Nozzle N12 (N12A & N12B): Level Transmitter Bridle

This nozzle pair connects to the Level Transmitter LZT-002A.

- Configuration: Two nozzles (A and B) forming a bridle loop.

- Valves:

  - N12A Connection: Connects to 2” Solenoid Valve HV38.

  - N12B Connection: Connects to 2” Solenoid Valve HV39.

- Instrumentation: LZT-002A (Level Transmitter).

- Piping & Isolation:

  - HV40: 1” Drain Valve. Located at the junction between the N12A connection and the LZT instrument.

  - Blind: A 2” blind flange is located before the drain valve.

  - HV41: 1” Gate Valve located on the connection line between N12A and N12B.

- Correlations:

  - This line connects to the same common header as L11 and N6: 1” PP01-512-PK-079-B03-F9-PP.

  - It connects specifically near the 1” Gate Valve HV52.

Nozzle N13: Pressure Transmitter (A)

This nozzle provides the first pressure reading for the voting logic system.

- Valve: 2” Solenoid Valve HV011.

- Instrumentation:

  - PZT-01A: Pressure Transmitter.

  - PI-1A: Pressure Indicator.

- Line: Connected directly via the solenoid.

Nozzle N14: Pressure Transmitter (B)

This nozzle provides the second pressure reading for the voting logic system.

- Valve: 2” Solenoid Valve HV12.

- Instrumentation:

  - PZT-01B: Pressure Transmitter.

  - PI-01B: Pressure Indicator.

- Note: The transcript has a conflicting statement at the end calling N14 a "3-in ball valve HV3". However, the detailed instrumentation section lists N14 as PZT-01B. HV3 is typically N17 (see next request if needed).

Nozzle N15: Pressure Transmitter (C)

This nozzle provides the third pressure reading for the voting logic system.

- Valve: 2” Solenoid Valve HV13.

- Instrumentation:

  - PZT-01C: Pressure Transmitter.

  - PI-01C: Pressure Indicator.

Nozzle N16: Pressure Control Transmitter

This nozzle connects the transmitter responsible for controlling the main pressure control valve (PV02) located on the outlet line.

- Valve: 2” Solenoid Valve HV14.

- Instrumentation:

  - PIT-02: Pressure Indication Transmitter.

  - PIC-02: Pressure Indicator Controller.

- Function: Controls the valve PV02 on the 8” HP Flare line (Line: 8” PP01-362-GF03-B01-E9-PP).

Nozzle N17: Maintenance / Vent

This is a standalone utility connection, separate from the complex instrument bridles.

- Nozzle Size: 3 inch.

- Valve: HV03 (3” Ball Valve).

- Detail: The line "just ends there," implying it is likely a vent or a blind flange connection used for maintenance access.

Nozzle N18 (N18A & N18B): Level Gauge Bridle

This nozzle pair is dedicated to the visual Level Gauge (LG-001). This was not covered in the N6–N10 or L11–N16 sections.

- Configuration: Two nozzles (A and B) forming a loop.

- Valves:

  - N18A Connection: Connects to 2” Solenoid Valve HV024.

  - N18B Connection: Connects to 2” Solenoid Valve HV025.

- Instrumentation: LG-001 (Level Gauge).

- Isolation Valves:

  - HV26: 1” Gate Valve (Top of gauge).

  - HV27: 1” Gate Valve (Bottom of gauge).

- Piping:

  - Line Number: 1” PP01-512-PK-076-B03-F9-PP.

  - Spec Change: A 1” Gate valve separates the spec B03-F9 from B01-M8.

- Correlation: This assembly connects to the Bottom Common Header (detailed below) alongside N7 and N10.

Common Instrumentation Lines (The "Missed" Correlations)

The transcript describes a complex web where multiple nozzles share common drain/fill headers. These are crucial for understanding how the nozzles interact.

1. The Bottom Common Header (Liquid Side)

This header collects connections from N7 (LIT-001), N10 (LIT-03), and N18 (LG-001).

- Header Line Number: 1” PP01-512-PK-54-B01-M8-PP.

- Function: It acts as a common manifold for the lower level instruments.

- Connection to Process: This header eventually correlates with the N3 liquid outlet piping network (specifically connecting near the 4” line 4” PP01-512-PK-06-B01-M8-PP).

2. The Top Common Header (Interface/Gas Side)

This header collects connections from L11 (LZT-002B), N12 (LZT-002A), and N6 (LZT-002C).

- Header Line Number: 1” PP01-512-PK-079-B03-F9-PP.

- Common Valves:

  - HV52: 1” Gate Valve (Isolation for the header).

  - HV56: 1” Drain Valve (Common drain for the upper instrument cluster).

- Connection Point: The individual lines from N6, L11, and N12 feed into this line before the drain valve HV56.

Equipment Design Data (Recap of Missed General Details)

To ensure "nothing is missed," here is the specific design data for the vessel itself mentioned at the start of the transcript:

- Equipment Tag: PP01-362-V01.

- Name: Fuel Gas Knockout Drum.

- Dimensions:

  - ID: 2200 mm.

  - Length (T/T): 5500 mm.

  - OD: 5500 mm (Transcript notes OD 5500, but standard conventions suggest this is the Length, consistent with the ID of 2200).

- Design Pressure: 14 bar (Transcript said "15 14 bar", confirmed by PSV set pressure).

- Design Temperature: -23°C to 72°C (implied range based on operating).

- Operating Pressure: 9 to 10.4 barge.

- Operating Temperature: 23°C to 72°C.

- Construction:

  - Material: Carbon Steel + 3mm Corrosion Allowance.

  - Internal: Epoxy Ceramic Coating.

  - Insulation: None.

- Trim Number: PP01 362 V1 TX1 V 03 F9.