1. Equipment Data & Specifications

Center your visualization on the vessel.

- Equipment Tag: PP01361 B2 (also referred to as 361 V002).

- Type: Scraper Receiver (Major/Minor Barrel design).

- Dimensions:

  - Major Barrel: 610 mm Internal Diameter (ID).

  - Minor Barrel: 508 mm ID (Note: Transcript said 58mm, but 508mm is physically required for the 20" connections described).

- Design Conditions:

  - Pressure: 45 barg.

  - Temperature: -29°C to 100°C.

- Operating Conditions:

  - Pressure: 10 to 11 barg.

  - Temperature: 20°C to 80°C.

- Material: Carbon Steel + 3 mm Corrosion Alloy (CA) internal cladding.

- Insulation: None (N O).

- Trim Number: PP01 361 V2 TX1 B 03 E7.

2. The Relief and Venting System (Top Nozzles)

Look to the top of the vessel for safety and pressure monitoring.

Nozzle N6 (Pressure Safety Valve - PSV):

- Start: A 3-inch Flange connects to the vessel.

- Line: 3" PP01 361 GF 018 B 03 E7-P.

- Reduction: The line reduces from 3-inch down to 1-inch.

- Component: PSV-02 (Pressure Safety Valve).

  - Spec Change: Piping class changes from 150 rating to 300 rating here.

- Discharge: After the PSV, a reducer expands the line from 2-inch to 4-inch.

- Drain/Isolation:

  - The discharge line features a Ball Valve (HV-26).

  - Between the reducer and HV-26, there is a 1/2-inch Drain Valve (HV-27).

- Exit Line: 4" PP01 500 VK 03 B 01 V8.

- Destination: Flare System (NPG PP01 PRP ID 21001).

Nozzle N4 (Instrumentation):

- Component: Connected to a 2-inch Solenoid Valve (HV-34).

- Instrument: Pressure Gauge (PG-004).

Nozzle N7 (Instrumentation):

- Component: Connected to a 2-inch Valve (HV-28).

- Instrument: Pressure Gauge (PG-003).

Body Instrumentation:

- Located physically between nozzles N7 and N9 is the Pig Signaler, tagged ZD-04 (Field) and ZI-004 (DCS).

3. The Small Bore Loop (Nozzles N1 & N9)

This is a loop system likely used for kicking or equalizing, involving two specific nozzles.

Nozzle N1:

- Size: 2-inch.

- First Valve: HV-38 (2" Ball Valve).

- Line: 2" PP01 361 GF 015 B 03 E7-P.

- Injection Point: Crucial Detail: Between the first valve (HV-38) and the second valve (HV-43), the line from Nozzle N9 enters here.

- Second Valve: HV-43 (2" Globe Valve).

- Instrument: Flow Element F402 (Field Instrument).

- Third Valve: HV-96 (2" Ball Valve).

- Spec Change: The line becomes 2" PP01 500 VK 02 B 01 B8.

- Destination: It connects to the Flare System line (3" PP01 500 VK 01 B 01 B8).

Nozzle N9:

- Size: 2-inch.

- Valve: HV-42 (2" Valve).

- Line: 2" PP01 361 GF 0018 B 03 E7-P.

- Destination: This line loops back and connects into the N1 Line (specifically between HV-38 and HV-43).

4. The Drain System (Bottom Nozzles N5, N12, N3)

Look at the bottom of the vessel.

The Main Drain Header (Merging N12 and N5):

- Nozzle N12:

  - Starts with a 4-inch Gate Valve (HV-14).

  - Line: 4" PP01 512 PK 61 B 03 E8-P. (Spec starts as B03E7, changes to B03E8 after valve).

- Nozzle N5:

  - Starts with a 4-inch Gate Valve (HV-92).

  - Line: 4" PP01 512 PK 0061 B 03 E8-PP.

- The Merge: N5 connects into the N12 line after HV-14.

- Common Drain Line:

  - Line: 4" PP01 512 PK 01 B 03 E8-P.

  - Drain Point: 1-inch Drain Valve (HV-15).

  - Isolation: 4-inch Normally Closed (NC) Ball Valve (HV-16).

  - Control: 4-inch Gate Valve (LCV-97).

  - Spec Change: Line becomes 4" PP01 512 PK 02 B 01 M8.

  - Destination: Condensate Storage Vessel (NPG PP01 PRP ID 0024 001).

Nozzle N3 (Sump Drain):

- Size: 2-inch.

- Valve: HV-35 (2" Ball Valve).

- Spec Change: Changes from B 03 E7 to B 01 H9.

- Drain: 1/2-inch Normally Closed Drain Valve (HV-36).

- Line: 2" PP01 567 GN 79 B 01 H9.

- Check Valve: HV-37 (2" Check Valve).

- Reducer: Reduces from 2-inch to 3/4-inch.

- Destination: Sump (SPN HN 028).

5. The Main Outlet & Barred Tee Complex (Nozzles N10 & N2)

This involves the massive 20-inch and 16-inch piping leaving the vessel.

Nozzle N10 (Main Scraper Outlet):

- Size: 20-inch.

- Line: 20" PP01 361 GF 0011 AP 15L-P.

- Valves:

  - HV-25 (20" Ball Valve).

  - HV-23 (20" Ball Valve).

  - Bleed: Between these two is a 2-inch Drain Valve (HV-24).

- Destination: This line flows into the Barred Tee.

The Barred Tee (BT) Connection:

- The Barred Tee merges the flow from the Scraper Receiver (N10) with the flow coming from the Scraper Launcher.

- Incoming from Launcher: Line 20" PP01 361 GF 07 AP 15L-PP (Details in Section 6).

- Branch Out: A 16-inch line exits the Barred Tee.

  - Line: 16" PP01 361 GF 014 B 03 F9-P.

  - Valve: MOV-002 (16-inch Control Ball Valve).

- Instrumentation on the 16" Line:

  - Temperature: TE-02 / TI-02 (2" connection).

  - Pressure: PI-04 / PIT-04 (1" connection) fitted with Solenoid Valve HV-46.

  - Corrosion Monitoring: 2" connections for CC02, CP002, CIT2, CE02.

- Destination: This 16-inch line goes to the Fuel Gas Knockout Drum (NPG PP01 PRP 0801).

Nozzle N2 (Bypass/Parallel Line):

- This nozzle connects the vessel to the Fuel Gas system (joining the 16" line described above).

- It splits into two parallel paths:

  - 8-inch Path: Contains two 8-inch Ball Valves (HV-29 and HV-31). Line: 8" PP01 361 GF 017 B 03 F9-P.

  - 2-inch Path: Contains a 2-inch Ball Valve (HV-32) and a 2-inch Globe Valve (HV-33).

- Termination: These paths merge and connect to the 16-inch line headed to the Fuel Gas Knockout Drum.

6. The Incoming Line (From Scraper Launcher)

This is the line feeding into the Barred Tee (upstream of the receiver).

- Line: 20" PP01 361 GF 07 AP 15L-PP.

- Main Valve: EZV-02 (20-inch Control Ball Valve).

- Pig Signal: Located between EZV-02 and the Barred Tee is ZI-3 / ZS-03.

Connections on this Incoming Line (Upstream of EZV-02):

- Flare Connection (3-inch):

  - Line: 3" PP01 361 GF 0061 B 03 E9-P.

  - Valves: HV-44 (3" Ball), HV-74 (3" Ball).

  - Drain: HV-45 (1/2" Drain Valve) located between the ball valves.

  - Instrument: FO5 (Flow Orifice).

  - Spec Change: Before the globe valve, spec is AP15L; it changes to B 03 E9.

  - Final Line: 3" PP01 500 VK 01 B 01 B8.

  - Destination: Flare System (NPG PP01 PRP 2101).

- Instrumentation Taps (2-inch):

  - Tap 1: HV-21 (2" Solenoid Valve) connecting to PZT-05, PI-05, PI-05 (SCADA).

  - Tap 2: HV-22 (2" Solenoid Valve) connecting to PIT-03, PI-03.