System Overview and Visualization

Imagine standing in front of a large industrial piping assembly. The centerpiece is the Scraper Receiver, a heavy pressure vessel designed to receive pipeline cleaning tools (pigs). The system consists of a massive 20-inch main pipeline, the receiver vessel itself, and a complex network of smaller bypass, drain, and vent lines managing pressure and fluid flow.

1. Equipment Specifications: The Scraper Receiver

- Tag: PP01361 P1 (also referred to as 361 V01).

- Dimensions: The vessel has a Major Barrel with a 610 mm Internal Diameter (ID) for handling the pig, and a Minor Barrel with a 508 mm ID connected to the pipeline.

- Material: Constructed from Carbon Steel with a 3 mm internal corrosion alloy lining.

- Conditions:

  - Design: Rated for 45 barg pressure with a temperature range of -29°C to +100°C.

  - Operating: Runs between 10 to 11 barg at 20°C to 80°C.

- Insulation: The vessel is uninsulated.

- Trim Number: PB01 361 B0 TX B 03 E7.

2. The Main Incoming Pipeline (20-Inch Line)

Everything begins with the main product flow arriving from the New PCB Station.

- Line: This is a 20-inch line (Tag: PP01 361 GF01 P 03 F9).

- Instrumentation: Immediately upon entering the diagram, the flow passes flow instrumentation:

  - FI-01 / FY-01 (Flow Indicator and Relay).

- Corrosion Injection: Downstream of the flow meter, there is a 2-inch injection point connecting to the Corrosion Inhibitor Injection Package (PB01 520 Z101). The line includes a specific valve arrangement (CP01, CC01, CI01) and connects via pump NPG PP01.

- Branching: This main 20-inch line continues past the receiver. However, to divert the pig into the receiver, flow is manipulated via the Kicker Line (Nozzle N1).

3. The Kicker Line (Nozzle N1)

This system diverts fluid to push the pig into the receiver. It taps off the main 20-inch line and enters the vessel at Nozzle N1.

- Line Size: 8-inch (Tag: PP01 361 GF 03 B 03 E7).

- Valve Arrangement (Double Block and Bleed):

  - The line passes through an 8-inch Ball Valve (HV-02).

  - It continues through a second 8-inch Ball Valve (HV-01).

  - Safety Bleed: Sandwiched between HV-02 and HV-01 is a 1-inch Drain Ball Valve (HV-03) used to depressurize the space between the valves.

- Bypass Loop: Parallel to this main kicker route, there is a smaller bypass for fine control:

  - It features a 2-inch Globe Valve (HV-05) and a 2-inch Ball Valve (HV-04).

  - Piping Spec Change: Before the Globe valve, the spec is B 03 E7; after the Globe valve, it changes to B 03 F9.

- Alternate Path (N11): There is also a 4-inch branch (Tag: GF05 B 03 E7) originating before HV-02. It passes through a 4-inch Valve (HV-10) and connects to Nozzle N11 (the receiving side of the barrel).

4. Venting, Equalization, and Relief Systems (N2, N7, N4)

To safely operate the receiver door, pressure must be managed via top-mounted nozzles.

- Nozzle N2 & N7 (Balance Line):

  - Nozzle N2 serves as the moving/pressurization side. It is a 2-inch line equipped with a 2-inch Ball Valve (HV-36).

  - Nozzle N7 connects directly to the N2 line. The N7 line (Tag: GF0010) includes a 2-inch Ball Valve (HV-11).

  - These lines merge. After the merge, the line flows through HV-06 (Ball Valve) and HV-07 (Globe Valve).

  - Control Valve: Branching off HV-07 is a control valve F401. Here, the piping spec changes from B 03 E7 to B 01 B8 (Tag: PP01 502...). This line vents to a Safe Location (SPN01).

- Nozzle N4 (Safety Relief):

  - This is a 3-inch line (GF0008) fitted with a blind flange initially.

  - It reduces from 3-inch to 1-inch to feed the Pressure Safety Valve (PSV-01).

  - PSV Details: Set pressure 45 barg, Size 1E2, Fireproof case.

  - Discharge: After the PSV, the line expands via a reducer (2" to 4") and vents to a safe location. This discharge line includes a 0.5-inch Drain Valve (HV-1).

5. Vessel Instrumentation (N3, N5 & Body)

- Nozzle N3: Fitted with a 2-inch Solenoid Valve (HV-08) and a Pressure Gauge connection (PG-02).

- Nozzle N5: Fitted with a 2-inch Solenoid Valve (HV-09) and a Pressure Gauge connection (PG-01).

- Pig Signaler: Mounted directly on the vessel is Z-02 / HZI-2, which signals when a pig has arrived in the barrel.

6. The Drain System (Nozzles N10 & N12)

Located at the bottom of the vessel to drain liquids into the closed drain system.

- Configuration:

  - Nozzle N10: 4-inch outlet with a Gate Valve. Spec changes from B 03 E7 to B 03 E8.

  - Nozzle N12: 4-inch outlet with a Gate Valve. Spec changes similarly.

  - These two lines (DK01 and DK02) merge into a common header.

- Drain Header:

  - The combined line passes through a 4-inch Gate Valve (HV-13).

  - Spec Change: After HV-13, the pipe spec changes to B 01 M8 (Tag: DK03).

  - There is a 0.5-inch drain point (HV-09) located near the main drain valves.

  - Termination: The line expands via a reducer (4" to 8") and routes to the Closed Drain System (2D DC1).

7. Main Outlet and Re-injection (Nozzle N9)

This is where the pig is retrieved, and the flow reconnects to the main line.

- Nozzle N9: The main barrel outlet.

- Isolation:

  - First, a 20-inch Ball Valve (HV-16).

  - Second, a 20-inch Ball Valve (HV-18).

  - Bleed: Between these two massive valves lies a 2-inch Drain Valve (HV-17).

- Barred Tee (Bar T): The line from N9 connects to the main pipeline via a Barred Tee. This special tee fitting prevents the pig from accidentally entering the bypass line.

- Main Line Control:

  - Downstream of the Barred Tee on the main line is the Motor Operated Valve/Control Valve EZV-01.

  - Pig Signaler: Between the Barred Tee and EZV-01, there is another Pig Signaler (Z-01 / ZI-01) to confirm the pig has passed or arrived.

- Final Output:

  - After passing EZV-01, the 20-inch line piping spec changes from AP15L to GF07 AP15L.

  - The line continues toward the next scraper section.

  - Solenoid Taps: Two final 2-inch connections are located here:

    - HV-19: Connects to PI-02 (DCS) and PIT-02 (Field) pressure transmitters.

    - HV-20: Connects to PZT-04 and PI-11.

    - Note: The small bore piping for these instruments changes spec to B 03 F9.