Scraper Receiver (P&ID Set).

1. Main Equipment Specifications

- Equipment Name: Scraper Receiver

- Tag Number: PP-01-361-V-001

- Trim Number: PP-01-361-V001-TX001-B03E7

- Dimensions:

  - Major Barrel: 610 mm Diameter

  - Minor Barrel: 508 mm Diameter

- Design Conditions:

  - Pressure: 45 Barg

  - Temperature: -29°C to 100°C

- Operating Conditions:

  - Pressure: 10 to 11 Barg

  - Temperature: 20°C to 80°C

- Material of Construction: Carbon Steel (NACE Standard) + 3mm Corrosion Allowance.

- Insulation: Not Insulated.

2. Nozzle N1 (8-Inch Line & Connections)

- Main Line Size: 8 Inch

- Line Number: 8"-PP-01-361-GF003-B03E7

- Valves & Components:

  - HV-0003: Ball Valve.

  - HV-0002: Ball Valve.

  - Drain Valve: 1-inch Ball Valve (HV-0003) located between HV-0002 and HV-0001 (Note: Speaker repeated 0003 for drain).

  - Parallel Connection: A 4-inch connection exists before HV-0002.

    - Line Number: 4"-PP-01-361-GF005-B03E7.

    - Valves: Connects to Globe Valve HV-0005 and Ball Valve HV-0004 (Both 2-inch).

    - Piping Spec Change: Before Globe Valve (B03E7)

    - →

After Globe Valve (B03F9).

  - Destination: Connects to N11 (Receiving side) and N2 (Moving side).

  - Connection to N11: Via 4-inch valve HV-0010.

3. Main Inlet Line (20-Inch from DS1)

- Origin: DS1 New PCB Station.

- Line Number: 20"-PP-01-361-GF001-B03F9.

- Instrumentation on Line:

  - FIT-001 / FY-001 / FI-001: Flow Indicator Transmitter suite.

- Chemical Injection: Connected to Corrosion Inhibitor Injection Pump (PP-01-520-Z-101) and Injection Package (PP-01-520-Z-101-Node-14).

  - Line: 20"-PP-01-361-GF001-B03F9.

  - Connection via 2-inch CP-001, CC-001, CIT-001, CE-001, CI-001.

- Main Valves:

  - MOV-001: 20-inch Motor Operated Control Valve.

  - HV-0016: 20-inch Ball Valve.

  - HV-0018: 20-inch Ball Valve.

  - HV-0017: 2-inch Drain Valve (located between HV-0016 & HV-0018).

    - Note: Corrosion allowance changes from 3mm to 6mm after HV-0018.

- Bypass/Control Branch (Barred Tee Connection):

  - EZV-001: 20-inch Control Valve.

  - Field Instruments: ZS-001 / ZI-001 (DCS Mounted).

  - Piping Spec: AP15L.

  - Line Number (After EZV-001): 20"-PP-01-361-GF007-AP15L.

4. Nozzle N2 & N7 (2-Inch Lines)

- Nozzle N2:

  - Size: 2-inch.

  - Valve: HV-0036 (2-inch Ball Valve).

- Nozzle N7:

  - Starts with 2-inch HV-0011.

  - Line Number: 2"-PP-01-361-GF0010-B03E7.

  - Connects with N2 line.

  - Line Number (N2 Side): 2"-PP-01-361-GF0039-B03E7.

- Control/Safety Path (N2/N7 Connection):

  - Valves: HV-0006 and Globe Valve HV-0007 (2-inch).

  - Control Valve: FV-001 (F4-001 mentioned in summary).

  - Piping Spec Change: B03E7

  - →

B01B8 (after control valve).

  - Destination Line: 2"-PP-01-502-V0001-B01B8 going to Safe Location (SPBN-001).

5. Nozzle N3 (Solenoid)

- Valve: HV-0008 (2-inch Solenoid Valve).

- Connection: PG-002 (Pressure Gauge).

6. Nozzle N4 (PSV / Safety Line)

- Size: 3-inch line.

- Configuration: Blind Flange

- →

Line 3"-PP-01-361-GF008-B03E7.

- Drain: HV-001 (1/2-inch Ball Valve) before PSV.

- Safety Valve:

  - PSV-001: Set pressure 45 Barg, Size 1E2, Fire Proof.

  - Inlet: Reducer 3" to 1".

  - Outlet: Reducer 2" to 4".

- Discharge:

  - Line: 4"-PP-01-502-V0000-B01B8.

  - Destination: Safe Location.

7. Nozzle N5 (Solenoid)

- Valve: HV-0009 (2-inch Solenoid Valve).

- Connection: PG-001 (Pressure Gauge).

8. Nozzle N9 (Vent/Instrumentation)

- Connection: Flange to 20-inch line.

- Valves:

  - HV-0016: 20-inch Ball Valve.

  - HV-0018: 20-inch Ball Valve.

  - Drain: HV-0017 (2-inch Drain Valve) located between the two 20" valves.

  - Note: Corrosion allowance changes: 3mm before HV-0018, 6mm after.

9. Nozzle N10 & N12 (Drain System)

- Nozzle N10:

  - Size: 4-inch.

  - Valve: Gate Valve.

  - Spec Change: B03E7

  - →

B03E8.

  - Line: 4"-PP-01-512-DK-001-B03E8.

- Nozzle N12:

  - Size: 4-inch.

  - Valve: HV-0009 (4-inch Gate Valve).

  - Line: 4"-PP-01-512-DK-002-B03E8.

  - Connects to: HV-0013 (4-inch Gate Valve).

- Discharge Path:

  - Reducer: 4-inch to 8-inch.

  - Destination: DC1 Closed Drain System (Line: 10-165-730-N-DG-01-PR-PID-007-002).

  - Drain Point: 3-inch connection for drain valve.

10. Instrumentation on Scraper Receiver

- Position Indication: ZS-002 / ZI-002 (Instrumentation connected to DCS/Field).

- Solenoid/Pressure Transmitters:

  - HV-0019 (2-inch Solenoid) connected to PIT-002.

  - HV-0020 (2-inch Solenoid) connected to PZT-0011 / PI-0011.

  - Piping Spec Change for Instruments: AP15L

  - →

B03F9 (on Solenoid start).

11. Scraper Passage Indicators (Pig Sigs)

- Instrumentation:

  - Z S-002 / ZI-002: Scraper Passage Indicator.

- Mounting: Connected directly to the receiver.

Comprehensive and detailed breakdown of the connections, piping patterns, valve arrangements, and instrumentation for the Scraper Receiver (Tag: PP-01-361-V-001) that were previously summarized.

1. Nozzle & Line Detailed Breakdown

N1: Kicker / Bypass Line (8-inch & 4-inch Pattern)

- Connection Pattern: This acts as the kicker line system.

- Start: Nozzle N1 connects to an 8-inch Line (8"-PP-01-361-GF003-B03E7).

- Valve Arrangement (Main Path):

  - The 8-inch line connects to HV-0002 (8-inch Ball Valve).

  - Downstream is HV-0001 (8-inch Ball Valve).

  - Double Block & Bleed Pattern: Between HV-0002 and HV-0001, there is a 1-inch Drain Ball Valve (HV-0003).

- Parallel/Branch Connection (4-inch Loop):

  - Before HV-0002, the line branches into a 4-inch Line (4"-PP-01-361-GF005-B03E7).

  - Valve: This line contains a 4-inch Globe Valve (HV-0010).

  - Connection Point: This 4-inch line connects to Nozzle N11 on the receiver.

- Spec Change: Piping spec changes from B03E7 to B03F9 at the connection interface.

N2 & N7: Equalization / Balance Lines

- N2 Configuration: 2-inch Nozzle

- →

Valve HV-0036 (2" Ball Valve)

→

Line 2"-PP-01-361-GF0039-B03E7.

- N7 Configuration: 2-inch Nozzle

- →

Valve HV-0011 (2" Ball Valve)

→

Line 2"-PP-01-361-GF0010-B03E7.

- Connection Pattern: The N7 line connects directly into the N2 line.

- Flow Path to Safe Location:

  - After N2 and N7 join, the line routes to a Control Valve Assembly.

  - Valves: It passes through HV-0006 and HV-0007 (Globe Valve).

  - Control Valve: FV-001 (referred to as F4-001 in audio).

  - Discharge: The line specs change from B03E7 to B01B8.

  - Destination: 2"-PP-01-502-V0001-B01B8

  - →

Safe Location (SPBN-001).

N10 & N12: Drain System Pattern

- Pattern: Two nozzles converging into a single closed drain header.

- N10 Path: 4-inch Nozzle

- →

Gate Valve

→

Line DK-001 (Spec B03E8).

- N12 Path: 4-inch Nozzle

- →

HV-0009 (4-inch Gate Valve)

→

Line DK-002 (Spec B03E8).

- Convergence: Both lines meet at HV-0013 (4-inch Gate Valve).

- Discharge:

  - Reducer: 4-inch expands to 8-inch.

  - Final Line: 8"-PP-01-502-DG-01-PR-PID-007-002.

  - Destination: DC1 Closed Drain System.

N9: Main Pipeline Connection (20-inch)

- Function: Connects the Scraper Receiver to the Main Gas Pipeline.

- Nozzle: N9 (Flanged connection).

- Line: 20-inch Line (Spec B03F9).

- Isolation Pattern (Double Block & Bleed):

  - HV-0016 (20-inch Ball Valve).

  - HV-0017 (2-inch Drain Ball Valve) acts as the bleed between the two main valves.

  - HV-0018 (20-inch Ball Valve).

- Corrosion Spec Change: The corrosion allowance increases from 3mm to 6mm after HV-0018 (Main pipeline side).

2. Main Inlet Line Details (From DS1)

- Source: DS1 New PCB Station.

- Main Line ID: 20"-PP-01-361-GF001-B03F9.

- Primary Isolation: MOV-001 (20-inch Motor Operated Valve).

- Control/Bypass:

  - Downstream includes EZV-001 (20-inch Control Valve).

  - Spec Change: B03F9 changes to AP15L (High pressure pipeline spec) after EZV-001.

- Barred Tee: The connection to the pig trap is via a Barred Tee to prevent the pig from entering the bypass line.

3. Safety & Relief System (N4)

- Nozzle N4: 3-inch connection.

- Arrangement: Blind Flange

- →

3-inch Line.

- Manual Bleed: HV-001 (1/2-inch Ball Valve) located upstream of the PSV.

- Pressure Safety Valve (PSV-001):

  - Inlet: Reducer 3"

  - →

1".

  - Outlet: Reducer 2"

  - →

4".

  - Specs: Set Pressure 45 Barg, Size 1E2, Fire Proof case.

- Discharge: 4-inch line to Safe Location (B01B8 spec).

4. Instrumentation & Injection Details

Chemical Injection (Corrosion Inhibitor)

- Location: On the main 20-inch inlet line.

- Line: 2-inch Injection connection.

- Equipment:

  - Injection Pump: PP-01-520-Z-101.

  - Injection Package: Node-14.

- Instrumentation Train:

  - FIT-001: Flow Indicator Transmitter.

  - CP-001 / CC-001: Corrosion Probe / Coupon.

  - CIT-001: Corrosion Inhibitor Transmitter.

Solenoid & Pressure Monitoring

- N3: 2-inch HV-0008 (Solenoid)

- →

PG-002 (Pressure Gauge).

- N5: 2-inch HV-0009 (Solenoid)

- →

PG-001 (Pressure Gauge). (Note: Audio uses tag HV-0009 for both N5 solenoid and N12 gate valve; likely a duplicate tag in the source).

- N9 Area / Main Line:

  - HV-0019 (2-inch Solenoid)

  - →

PIT-002 (Pressure Transmitter).

  - HV-0020 (2-inch Solenoid)

  - →

PZT-0011 / PI-0011.

  - HV-0020 Spec Change: Piping changes from AP15L to B03F9 for the instrument connection.

Scraper Passage Indicators (Pig Sigs)

- Tags: ZS-002 and ZI-002.

- Mounting: Direct mount on the barrel (Instrumentation Connection Z1-002).

- Interface: Connected to Field (ZI) and DCS (ZS).

5. Instrumentation Connections (Small Bore)

- N13 / N14 (Likely):

  - HV-0015 (2-inch Solenoid) connects to PIT-001.

  - HV-0008 is also associated with PG-002 via connection PG-0002.